Magnetic Materials

- Three materials are known to have magnetic susceptibilities of

- 2x10-4

- -3x10-5

- 1x105

Indicate which of these materials is paramagnetic, which one is ferromagnetic, and which one is diamagnetic.  Explain your answer.

- Paramagnetic (small positive susceptibility)

- Diamagnetic (negative susceptibility)

- Ferromagnetic (very large susceptibility)

- Calculate the molar susceptibility for a He atom.  (Assume an average electron radius of 0.58Å).

- In an H-field of 106A/m, if an atom has a magnetic dipole moment of three Bohr magnetons and the moment rotates from antiparallel to the field to parallel to the field,

  - By how much does its energy decreases?

  - At T=300K, what fraction of KbT is this energy?

- Consider a paramagnetic material with a susceptibility of 2x10-4, Calculate the curie constant C at 300K.

- Consider a monoatomic H gas and assume that only the 1s level is available for the electrons.  A magnetic field of 1T is applied to this gas.  Calculate the population of each of the two spin states.

- An H-field of 20 A/m is applied to a ferromagnet with a saturation magnetization of 1.4x106 A/m.  If the magnetic field inside the material is 0.5 Testa

  - What percentage of the domains are aligned to the field

  - What is the magnetic susceptibility

  - What H field should be applied to this material to align all the domains to the field

- Consider a material for which the anisotropy field is 5x106 A/m with a saturation magnetization of 1.4x106 A/m.

  - What is the magnetic anisotropy constant

  - What is the component of the magnetization parallel to the field for H=2x105 A/m

  - What for H=5x106 A/m

  - What for H=6x106 A/m

- For a hypothetical material, the exchange integral is JE=12 meV, the anisotropy constant per unit of volume is K=1.5x104 J/m3 and the wall energy per unit of area is 1x10-3J/m2.  What is the wall thickness? S=1/2